Basic Operation

The basic operation of ignition system is described in this video:

https://www.youtube.com/watch?v=OMLSNwQiiKg

Points-Style

The basic architecture for ignition systems is best illustrated in the original points-style ignition (as described in the video linked above).

Typically, the only wires to the distributor are +12V from the coil and ground.

Electronic Ignition

Electronic ignition systems function analogously as the points-style but with key mechanical components replaced with electronic ones.  Specifically, the points and condenser are replaced with solid-state components.  At IIS, three types of electronic distributors are used.

- HEI (High Energy Ignition) –

  - IIS uses the HEI variation featuring an integrated coil (on the cap) and vacuum advance.

  - Used primarily on engines equipped with mechanical fuel systems.

  - Vacuum advance remains capped.

  - The connector on the cap has two connections: one labeled “+”, the other “tach”.

  - Only requires +12V power to operate.

  - The tachometer signal must be conditioned and clamped by the receiving device.

- EST (Electronic Spark Timing) –

  - A type of HEI that uses an electronic control module to control timing rather than vacuum or centrifugal mechanisms.

  - This distributor features a speed (tachometer) output which allows the “smart” governor (L-Series) to be used without a separate ECM.

  - It is the most common distributor used at IIS.

  - It requires an external coil.

  - The most notable feature about its appearance is the two connectors.

The following illustration shows typical wiring to EST distributor; but wiring to the 4-position connector varies per application.

Referring to the 4-position connector in the illustration above:

- Pin B receives the “override (cranking) signal”  which “is zero volts during cranking (less than 400 rpm or 5 to 15 seconds), then there is 5 volts on this wire after the engine starts to signal the 8-pin module that it should use” the external signal (if provided) to control timing.

- Pin D is used to receive this external timing control if an ECM is utilized for spark control.

- “The module converts the AC signal from the variable reluctor pick-up in the distributor to a 'square wave' tach signal “.  This signal is transmitted on Pin C.

- Pin A is a transducer ground circuit.

This web page offers detailed technical information about HEI (and EST) modules:

General Motors HEI Ignition Control (megamanual.com)

Here are a couple of useful videos further explaining EST distributors:

EST Shop Manual

Marine Delco EST Distributor - YouTube

- HVS (High Voltage Switch, a.k.a. Flat Top) –

  - This is an HEI distributor that is characterized by the low-profile, horizontal spark plug wire terminals.

  - Utilizes a 3-position connector.

  - Requires ignition coil with integrated control module.